In the design of the program, we need to use the OneWrie library. This library is not part of the Arduino basic library. The OneWrie library and the DallasTemperature library will be attached to the post. The compressed files can be extracted into the libraries file.

Experiment with the OneWrie library function alone.

== interface definition ==

(The pin definition of each model DS1820)

==Arduino wiring method ==

Wiring method: Red line (VCC, 3.3V-5.5V) and black line (GND) pins are connected to Arduino 5V and GND respectively.

The yellow line (DATA signal line) is connected to the Arduino PIN 10. The resistance between the black line and the yellow line is 4.7K.。

Wiring method: VD and GND pins are connected to Arduino 5V and GND respectively, DQ is connected to Arduino PIN 10. The resistance connected to it is 10K.

(The pull-up resistor is already built in the temperature and humidity sensor module, no need to connect the resistor)

Wiring method: Red line (VCC, 3.3V-5.5V) and black line (GND) pins are connected to Arduino 5V and GND respectively, and yellow line (DATA signal line) is connected to Arduino PIN 10.

The resistance between the red line and the yellow line is 4.7K.

(The temperature and humidity sensor probe needs to be connected to a 4.7KΩ pull-up resistor)

Probe parameters:

High quality stainless steel tube package waterproof, moisture proof, antirust

Stainless steel head (6*50mm), lead 100cm

Each probe is rigorously tested

3.0V ~ 5.5V power supply

9 to 12 adjustable resolution

The measurement temperature range is -55 ° C to +125 °C. Fahrenheit is 67 ° F to 257 ° F -10 ° C to +85 ° C with an accuracy of ±0.5 ° C

Use range:

1. This product is suitable for temperature measurement and control in freezer, granary, storage tank, telecommunications room, electric machine room, cable trunking, etc.

2. Temperature measurement and control of industrial equipment such as bearing bush, cylinder block, textile machine, air conditioner, etc.

3. Automotive air conditioners, refrigerators, freezers, and medium and low temperature drying ovens.

4. Heating/refrigeration pipe heat metering, central air conditioning household heat metering and industrial temperature measurement and control

Output leads: red (VCC), yellow (DATA), black (GND)